UDС 629.038

Nekrasov V.G., Kuanishev M.K., Kaukarov A.K.,

Muhtarov A.T. Optimum construction of internal combustion engine // Internal combustion engines. –

2006. – № 1. – P. 32-36.

The considered approach to development of the optimum

car engine, mainly, for car of the compact class,

which module can serve the base for making the engines

whole capacity row of the car engines. The actions, raising

thermal, indicative and mechanical efficiency, as

well as need to utilization of heat exhaust gas has served

as a reference for development two cylinders module

with opposition location cylinders on base crankshaftlink

of the mechanism of the transformation of the motion.

The engine of the diesel type, with continued of

expansion. The cylinder is executed in the manner of

two-layer bushing with internal layer from material with

low heat conductivity. The mechanism gas distribution is

applying of the rotary valve type, at role of rotary valve

executes the revolving cylinder with lid of the bushing.

For reduction of the losses on friction in mechanism gas

distribution and for compaction of piston into cylinder is

applying composition antifriction material with use the

graphite, as well as applying bearings in kinematics

pair of mechanism of the transformation of the motion

with lowered of friction. Il. 3. Bibliogr. 3 names.